Specification of floor swing door operator

| - Item | - Parameter | - Remark |
| --- | --- | --- |
| - Working voltage | - 220VAC±15% 50Hz |  |
| - Motor operating voltage | - 24VDC |  |
| - Power of motor | - 48W |  |
| - Rated output torque | - 56Nm |  |
| - Angular speed | - 48°/s |  |
| - Maximum opening Angle | - ±90° |  |
| - Maximum length of door leaf | - 1.2m (single door) |  |
| - Maximum weight of door leaf | - 250Kg |  |
| - Thermal protection on motor winding | - F Class 155℃ |  |
| - Operating frequency | - Continuous running below 70℃ |  |
| - Operating ambient temperature | - -25-70℃ |  |
| - Working environment | - Indoor or sheltered outdoor environments |  |
| - Running noise | - ≤60dB |  |
| - Temperature rise | - ≤15℃ |  |
| - Weight of equipment packaging | - 7.5kg |  |
| - Body size (length * width * thickness) | - 350*150*75mm |  |
| - Packing size | - 425×285×210mm |  |